|  | Pimpri Chinchwad Education Trust’s Pimpri Chinchwad College of Engineering |
| --- | --- |
| Experiment 6 Experimentation details |  |

Department: E&TC                      Academic Year: 2025-26                       Semester-I

Class: B.Tech E&TC                   Course: VLSI Design Lab

Name:

Div and batch:

Roll No:

Title of the Experiment:

Model and verify 4-bit Universal shift using VHDL and implement on FPGA and evaluate power and timing performance

Software Requirements:  Xilinx 14.7 ISE Design Suite

Hardware Requirements: Xilinx Spartan-6 XC6SLX45 FPGA

VHDL Code for Simulation

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_arith.ALL;

use IEEE.STD_LOGIC_unsigned.ALL;

entity usr is

Port ( Clk, rst, sin : in  STD_LOGIC;

m : in  STD_LOGIC_VECTOR (1 downto 0);

pin : in  STD_LOGIC_VECTOR (3 downto 0);

sout : out  STD_LOGIC;

pout : out  STD_LOGIC_VECTOR (3 downto 0));

end usr;

architecture Behavioral of usr is

signal p1,p2:std_logic_vector(3 downto 0);

signal s1:std_logic;

begin

process(clk,rst,pin,sin,m)

begin

if(rst='1') then

p1<="0000";

p2<="0000";

s1<='0';

elsif (clk'event and clk='1') then

if(m="00") then     --SISO

p1(0)<=sin;

p1(1)<=p1(0);

p1(2)<=p1(1);

p1(3)<=p1(2);

s1<=p1(3);

elsif(m="01") then   --SIPO

p1(0)<=sin;

p1(1)<=p1(0);

p1(2)<=p1(1);

p1(3)<=p1(2);

p2<=p1;

elsif(m="10") then   --PIPO

p1<=pin;

p2<=p1;

else                 --PISO

p1<=pin;

s1<=p1(3);

end if;

end if;

end process;

pout<=p2;

sout<=s1;

end Behavioral;

RTL Schematic

VHDL Test Bench

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--USE ieee.numeric_std.ALL;

ENTITY usr_tb IS

END usr_tb;

ARCHITECTURE behavior OF usr_tb IS

-- Component Declaration for the Unit Under Test (UUT)

COMPONENT usr

PORT(

Clk : IN  std_logic;

rst : IN  std_logic;

sin : IN  std_logic;

m : IN  std_logic_vector(1 downto 0);

pin : IN  std_logic_vector(3 downto 0);

sout : OUT  std_logic;

pout : OUT  std_logic_vector(3 downto 0)

);

END COMPONENT;

--Inputs

signal Clk : std_logic := '0';

signal rst : std_logic := '0';

signal sin : std_logic := '0';

signal m : std_logic_vector(1 downto 0) := (others => '0');

signal pin : std_logic_vector(3 downto 0) := (others => '0');

--Outputs

signal sout : std_logic;

signal pout : std_logic_vector(3 downto 0);

-- Clock period definitions

constant Clk_period : time := 20 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: usr PORT MAP (

Clk => Clk,

rst => rst,

sin => sin,

m => m,

pin => pin,

sout => sout,

pout => pout

);

-- Clock process definitions

Clk_process :process

begin

Clk <= '0';

wait for Clk_period/2;

Clk <= '1';

wait for Clk_period/2;

end process;

rst_process :process

begin

rst <= '1';

wait for Clk_period;

rst <= '0';

wait;

end process;

-- Stimulus process

stim_proc: process

begin

pin<="1001";

sin<='1';

m<="00”;     -- SISO

wait for 120 ns;

pin<="1101";

sin<='1';

m<="01";   -- SIPO

wait for 120 ns;

pin<="1101";   --PIPO

m<="10";

wait for 120 ns;

pin<="0100";    -- PISO

m<="11";

wait;

end process;

END;

Simulation Results

VHDL Code for Implementation

Plz note:

1. Clock freq of board= 8MhZ.   So, a process is added to generate a slow clock of freq = 1 Hz

2. for slowing down the clk from 8 MHz to 1 Khz, a temp counter of size 23 bits is

introduced for generating delay

VHDL Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_arith.ALL;

use IEEE.STD_LOGIC_unsigned.ALL;

entity usr is

Port ( Clk,rst,sin : in  STD_LOGIC;

m : in  STD_LOGIC_VECTOR (1 downto 0);

pin : in  STD_LOGIC_VECTOR (3 downto 0);

sout : out  STD_LOGIC;

pout : out  STD_LOGIC_VECTOR (3 downto 0));

end usr;

architecture Behavioral of usr is

signal p1,p2:std_logic_vector(3 downto 0);

signal s1:std_logic;

signal temp:std_logic_vector(22 downto 0);

signal sclk:std_logic;

begin

p11: process(clk,rst)

begin

if(rst='1')then temp<=(others=>'0');

elsif(clk'event and clk='1') then

temp<= temp + 1;

end if;

end process;

sclk<=temp(22);

p22:process(sclk,rst,pin,sin,m)

begin

if(rst='1') then

p1<="0000";

p2<="0000";

s1<='0';

elsif (sclk'event and sclk='1') then

if(m="00") then     --SISO

p1(0)<=sin;

p1(1)<=p1(0);

p1(2)<=p1(1);

p1(3)<=p1(2);

s1<=p1(3);

elsif (m="01") then   --SIPO

p1(0)<=sin;

p1(1)<=p1(0);

p1(2)<=p1(1);

p1(3)<=p1(2);

p2<=p1;

elsif(m="10") then   --PIPO

p1<=pin;

p2<=p1;

else                 --PISO

p1<=pin;

s1<=p1(3);

end if;

end if;

end process;

pout<=p2;

sout<=s1;

end Behavioral;

Implementation Constraint File (UCF File)

NET "CLK"   LOC = "L15"; # Bank = 1, Pin name = IO_L42P_GCLK7_M1UDM, Type = GCLK, Sch name = GCLK

# onBoardLeds

NET "pout<0>" LOC = "U18"; # Bank = 1, Pin name = IO_L52N_M1DQ15,  Sch name = LD0

NET "pout<1>" LOC = "M14"; # Bank = 1, Pin name = IO_L53P,  Sch name = LD1

NET "pout<2>" LOC = "N14"; # Bank = 1, Pin name = IO_L53N_VREF,  Sch name = LD2

NET "pout<3>" LOC = "L14"; # Bank = 1, Pin name = IO_L61P,  Sch name = LD3

NET "Sout" LOC = "M13"; # Bank = 1, Pin name = IO_L61N,  Sch name = LD4

# onBoard SWITCHES

NET "pin<0>" LOC = "A10"; # Bank = 0, Pin name = IO_L37N_GCLK12,  Sch name = SW0

NET "pin<1>" LOC = "D14"; # Bank = 0, Pin name = IO_L65P_SCP3,  Sch name = SW1

NET "pin<2>" LOC = "C14"; # Bank = 0, Pin name = IO_L65N_SCP2,  Sch name = SW2

NET "pin<3>" LOC = "P15"; # Bank = 1, Pin name = IO_L74P_AWAKE_1,  Sch name = SW3

NET "sin" LOC = "P12"; # Bank = 2, Pin name = IO_L13N_D10,  Sch name = SW4

NET "rst" LOC = "R5";  # Bank = 2, Pin name = IO_L48P_D7,  Sch name = SW5

NET "m<0>" LOC = "T5";  # Bank = 2, Pin name = IO_L48N_RDWR_B_VREF_2, Sch name = SW6

NET "m<1>" LOC = "E4";  # Bank = 3, Pin name = IO_L54P_M3RESET,  Sch name = SW7

Hardware Snapshots

Digilent ATLYS Spartan 6 FPGA Board