Experiment 7: Model and verify FIFO using VHDL and implement on FPGA and evaluate power and timing performance

VHDL Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_logic_arith.ALL;

use IEEE.STD_logic_unsigned.ALL;

use IEEE.NUMERIC_STD.ALL;

entity fifo is

generic (depth : integer := 8);  --depth of fifo

port (    clk : in std_logic;

reset : in std_logic;

enr : in std_logic;   --enable read,should be '0' when not in use.

enw : in std_logic;    --enable write,should be '0' when not in use.

data_in : in std_logic_vector (2 downto 0);     --input data

data_out : out std_logic_vector(2 downto 0);    --output data

fifo_empty : out std_logic;     --set as '1' when the queue is empty

fifo_full : out std_logic     --set as '1' when the queue is full

);

end fifo;

architecture Behavioral of fifo is

type memory_type is array (0 to depth-1) of std_logic_vector(2 downto 0);

signal memory : memory_type :=(others => (others => '0'));   --memory for queue.

signal readptr,writeptr : integer := 0;  --read and write pointers.

signal empty,full : std_logic := '0';

signal temp:std_logic_vector(27 downto 0);

signal sclk:std_logic;

signal dataout:std_logic_vector(2 downto 0)

begin

fifo_empty <= empty;

fifo_full <= full;

p11: process(clk,reset)

begin

if(reset='1')then temp<=(others=>'0');

elsif(clk'event and clk='1') then

temp<= temp + 1;

end if;

end process;

sclk<=temp(27);

p12:process(sclk,reset)

--this is the number of elements stored in fifo at a time.

--this variable is used to decide whether the fifo is empty or full.

variable num_elem : integer := 0;

begin

if(reset = '1') then

dataout <= (others => '0');

empty <= '0';

full <= '0';

readptr <= 0;

writeptr <= 0;

num_elem := 0;

elsif(rising_edge(sclk)) then

if(enr = '1' and empty = '0') then  --read

dataout <= memory(readptr);

readptr <= readptr + 1;

num_elem := num_elem-1;

end if;

if(enw ='1' and full = '0') then    --write

memory(writeptr) <= data_in;

writeptr <= writeptr + 1;

num_elem := num_elem+1;

end if;

--rolling over of the indices.

if(readptr = depth-1) then      --resetting read pointer.

readptr <= 0;

end if;

if(writeptr = depth-1) then        --resetting write pointer.

writeptr <= 0;

end if;

--setting empty and full flags.

if(num_elem = 0) then

empty <= '1';

else

empty <= '0';

end if;

if(num_elem = depth) then

full <= '1';

else

full <= '0';

end if;

end if;

end process;

data_out<=dataout;

end Behavioral;

UCF File:

NET "reset" LOC = "E4";

NET "CLK" LOC ="L15";

NET "enr" LOC ="T5";

NET "enw" LOC ="R5";

NET "data_in<0>" LOC ="A10";

NET "data_in<1>" LOC ="D14";

NET "data_in<2>" LOC ="C14";

NET "data_out<0>" LOC = "U18"; # Bank = 1, Pin name = IO_L52N_M1DQ15,  Sch name = LD0

NET "data_out<1>" LOC = "M14"; # Bank = 1, Pin name = IO_L53P,  Sch name = LD1

NET "data_out<2>" LOC = "N14"; # Bank = 1, Pin name = IO_L53N_VREF,  Sch name = LD2

NET "fifo_empty" LOC = "P16"; # Bank = 1, Pin name = IO_L61P,  Sch name = LD3

NET "fifo_full" LOC = "D4";

Simulatiion:

Implementation On FPGA Board:

Write & Read 101

Write & Read 111