Experiment 8: Model and verify Modulo-N-Counter (MOD_N) using VHDL and implement on FPGA and evaluate power and timing performance.

Program:

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity modN_counter is

generic(n: integer := 128);

port(

clk : in STD_LOGIC;

reset : in STD_LOGIC;

dout : out STD_LOGIC_VECTOR(6 downto 0)

);

end modN_counter;

architecture A_modN of modN_counter is

signal temp:std_logic_vector(22 downto 0);

signal sclk:std_logic;

begin

p11: process(clk,reset)

begin

if(reset='1')then temp<=(others=>'0');

elsif(clk'event and clk='1') then

temp<= temp + 1;

end if;

end process;

sclk<=temp(22);

counter : process (sclk,reset) is

variable m : integer range 0 to (n-1) := 0;

begin

if (reset='1') then

m := 0;

--elsif (rising_edge (clk)) then

elsif(sclk'event and sclk='1') then

m := m + 1;

end if;

if (m=(n)) then

m := 0;

end if;

dout<= conv_std_logic_vector (m,7);

end process counter;

end A_modN;

UCF File:

NET "reset" LOC = "E4";

NET "CLK" LOC ="L15";

NET "dout<0>" LOC = "U18"; # Bank = 1, Pin name = IO_L52N_M1DQ15,  Sch name = LD0

NET "dout<1>" LOC = "M14"; # Bank = 1, Pin name = IO_L53P,  Sch name = LD1

NET "dout<2>" LOC = "N14"; # Bank = 1, Pin name = IO_L53N_VREF,  Sch name = LD2

NET "dout<3>" LOC = "L14"; # Bank = 1, Pin name = IO_L61P,  Sch name = LD3

NET "dout<4>" LOC = "M13"; # Bank = 1, Pin name = IO_L61N,  Sch name = LD4

NET "dout<5>" LOC = "D4"; # Bank = 1, Pin name = IO_L61P,  Sch name = LD3

NET "dout<6>" LOC = "P16"; # Bank = 1, Pin name = IO_L61P,  Sch name = LD3

#NET "dout<7>" LOC = "N12";

Simulation Results:

Implementation on FPGA Board: